ECA04 – ANALOG CIRCUITS

CO1 – INVESTIGATIVE ASSIGNMENT

QUESTION:

Analyze Feedback amplifier configurations to evaluate their impact on gain, bandwidth, input impedance, output impedance and overall amplifier performance ?

| Student Details | Information |
| --- | --- |
| Name | Y. Balu Prasanth |
| Register Number | 192512179 |
| Programme | B.E. – Electronics and Communication Engineering |
| Institution | SIMATS Engineering, Saveetha Institute of Medical and Technical Sciences, Chennai |
| Course | ECA04 – Analog Circuits |
| Course Outcome | CO1 – Analyze feedback amplifier configurations |

Submitted in partial fulfilment of the course requirements

# Introduction

An amplifier is used to increase the level of a weak signal. However, an amplifier with high open-loop gain may have unsuitable input and output resistance and a limited bandwidth. Negative feedback returns a fraction of the output signal to the input in an opposing direction. This reduces the overall gain, but it can make the gain more stable, increase bandwidth, increase input resistance and reduce output resistance when the correct feedback topology is selected.

The purpose of this investigation is to compare the four basic feedback configurations and calculate whether a voltage-series feedback amplifier can meet the required specifications. All calculations below use the data and standard equations stated in the assignment. The feedback equations are treated as an ideal small-signal model; actual hardware may differ because of loading, component tolerances and amplifier limitations.

# Given specifications

| Parameter | Symbol | Value |
| --- | --- | --- |
| Open-loop voltage gain | A | 100 |
| Input resistance | Ri | 10 kΩ |
| Output resistance | Ro | 10 kΩ |
| Lower cutoff frequency | fL | 100 Hz |
| Upper cutoff frequency | fH | 100 kHz |
| Load resistance | RL | 10 kΩ |
| Required closed-loop gain | Af | 20 |
| Required input resistance | Rif | ≥ 50 kΩ |
| Required output resistance | Rof | ≤ 2 kΩ |
| Required lower cutoff frequency | fLf | ≤ 25 Hz |
| Required upper cutoff frequency | fHf | ≥ 400 kHz |

# Equations used

• Open-loop gain in decibels: AdB = 20 log10(A).

• Input current: Ii = Vi / Ri.

• Loaded output voltage: VL = Vo × RL / (Ro + RL).

• Negative-feedback desensitivity factor: D = 1 + Aβ.

• Closed-loop gain: Af = A / (1 + Aβ) = A / D.

• Voltage-series feedback: Rif = RiD and Rof = Ro / D.

• For the standard single-pole approximation: fLf = fL / D and fHf = fH D.

• Bandwidth: BW = fH − fL.

# Stage 1 – Interpret and analyse the design requirements

### 1. Calculate the open-loop voltage gain in dB

Formula: AdB = 20 log10(A)

Substitution: AdB = 20 log10(100)

Answer:   ⇒  40 dB

### 2. Calculate output voltage for a 100 mV input

Given: Vi = 100 mV = 0.1 V

Formula: Vo = AVi = 100 × 0.1 V

Answer:   ⇒  10 V (ideal, before load division)

### 3. Calculate input current for a 100 mV input

Formula: Ii = Vi / Ri = 0.1 V / 10,000 Ω

Answer:   ⇒  10 μA

### 4. Determine voltage available across the 10 kΩ load

Formula: VL = Vo × RL / (Ro + RL)

Substitution: VL = 10 × 10 kΩ / (10 kΩ + 10 kΩ)

Answer:   ⇒  5 V

Because the load and amplifier output resistance are equal, the load receives half of the ideal 10 V output.

### 5. Determine the lower and upper cutoff frequencies

Lower cutoff: fL = 100 Hz

Upper cutoff: fH = 100 kHz

### 6. Calculate the original bandwidth

Formula: BW = fH − fL = 100,000 − 100 Hz

Answer:   ⇒  99,900 Hz = 99.9 kHz

### 7. Calculate loaded voltage gain considering Ro and RL

Formula: AL = A × RL / (Ro + RL)

Substitution: AL = 100 × 10 / (10 + 10)

Answer:   ⇒  50 V/V (approximately 33.98 dB)

### 8. Compare open-loop performance with the required specifications

| Parameter | Original / loaded result | Requirement | Evaluation |
| --- | --- | --- | --- |
| Gain | 100 open-loop; 50 loaded | 20 | Not met |
| Input resistance | 10 kΩ | ≥ 50 kΩ | Not met |
| Output resistance | 10 kΩ | ≤ 2 Kω | Not met |
| Lower cutoff | 100 Hz | ≤ 25 Hz | Not met |
| Upper cutoff | 100 kHz | ≥ 400 kHz | Not met |

Stage 1 conclusion: Without feedback, the amplifier fails all five listed design targets. Negative feedback and a suitable topology are needed to obtain the required gain, resistance and bandwidth.

# Stage 2 – Investigate and select the feedback configuration

The assignment asks for a comparison using A = 100 and β = 0.04. The desensitivity factor is:

D = 1 + Aβ: D = 1 + 100(0.04) = 1 + 4

Answer:   ⇒  D = 5

The four configurations are distinguished by how the output is sampled and how the feedback signal is mixed at the input. For voltage amplifiers, voltage sampling is useful for reducing output resistance. Series mixing increases input resistance, so voltage-series feedback is the best match to the required resistance behaviour.

| Configuration | Input mixing / output sampling | Input resistance relationship | Output resistance relationship | Calculated Rif | Calculated Rof | Suitable? |
| --- | --- | --- | --- | --- | --- | --- |
| Voltage-series | Series / voltage | Rif = RiD | Rof = Ro/D | 50 kΩ | 2 kΩ | Yes |
| Voltage-shunt | Shunt / voltage | Rif = Ri/D | Rof = Ro/D | 2 kΩ | 2 kΩ | No: input too low |
| Current-series | Series / current | Rif = RiD | Rof = RoD | 50 kΩ | 50 kΩ | No: output too high |
| Current-shunt | Shunt / current | Rif = Ri/D | Rof = RoD | 2 kΩ | 50 kΩ | No: both fail |

### 9. State the expected effect on input resistance

Series mixing increases input resistance by the factor D. Shunt mixing decreases input resistance by the factor D.

### 10. State the expected effect on output resistance

Voltage sampling decreases output resistance by D. Current sampling increases output resistance by D.

### 11. Calculate input and output resistance using the appropriate relationships

Voltage-series input resistance: Rif = RiD = 10 kΩ × 5

Answer:   ⇒  50 kΩ

Voltage-series output resistance: Rof = Ro/D = 10 kΩ/5

Answer:   ⇒  2 kΩ

### 12. Compare the results with the required specifications

The voltage-series configuration gives Rif = 50 kΩ and Rof = 2 kΩ. Both values meet the specified limits exactly. The other three configurations fail at least one resistance requirement.

Design Decision 1: Voltage-series feedback is selected because it combines series input mixing with voltage output sampling. This produces high input resistance and low output resistance, which are the required properties of this voltage amplifier.

# Stage 3 – Gain, feedback factor and desensitivity

### 13. Determine β for Af = 20 and A = 100

Starting equation: Af = A/(1 + Aβ)

Rearrangement: β = (A/Af − 1)/A

Substitution: β = (100/20 − 1)/100 = 4/100

Answer:   ⇒  β = 0.04

### 14. Determine Aβ

Calculation: Aβ = 100 × 0.04

Answer:   ⇒  Aβ = 4

### 15. Determine 1 + Aβ

Calculation: 1 + Aβ = 1 + 4

Answer:   ⇒  D = 5

### 16. Calculate feedback voltage when output voltage is 2 V

Formula: Vf = βVo = 0.04 × 2 V

Answer:   ⇒  Vf = 0.08 V = 80 mV

### 17. Calculate percentage reduction in gain

Formula: Percentage reduction = (A − Af)/A × 100%

Substitution: (100 − 20)/100 × 100%

Answer:   ⇒  80%

### 18. Calculate desensitivity factor

Formula: S = A/Af = 1 + Aβ = 100/20

Answer:   ⇒  S = 5

A desensitivity factor of 5 means the closed-loop gain is less sensitive to changes in the forward gain than the open-loop gain.

## Gain variation investigation

For the comparison, β remains fixed at 0.04.

When A = 90: Af = 90/[1 + 90(0.04)] = 90/4.6

Result:   ⇒  Af = 19.565

Percentage variation: (19.565 − 20)/20 × 100%

Result:   ⇒  −2.17%

When A = 110: Af = 110/[1 + 110(0.04)] = 110/5.4

Result:   ⇒  Af = 20.370

Percentage variation: (20.370 − 20)/20 × 100%

Result:   ⇒  +1.85%

| Open-loop gain A | Closed-loop gain Af | Variation from nominal 20 |
| --- | --- | --- |
| 90 | 19.565 | −2.17% |
| 100 | 20.000 | 0% |
| 110 | 20.370 | +1.85% |

Engineering interpretation: The open-loop gain changes by ±10%, while the closed-loop gain changes by approximately −2.17% to +1.85%. Negative feedback therefore improves gain stability.

# Stage 4 – Effect of feedback on bandwidth

For this stage, the assignment's standard single-pole relationships are used, with fL = 100 Hz, fH = 100 kHz and D = 5.

### 19. Calculate the new lower cutoff frequency

Formula: fLf = fL/D = 100/5

Answer:   ⇒  20 Hz

### 20. Calculate the new upper cutoff frequency

Formula: fHf = fH D = 100 kHz × 5

Answer:   ⇒  500 kHz

### 21. Calculate the new bandwidth

Formula: BWf = fHf − fLf = 500,000 − 20 Hz

Answer:   ⇒  499,980 Hz = 499.98 kHz

### 22. Calculate lower-frequency improvement factor

Calculation: fL/fLf = 100/20

Answer:   ⇒  5

### 23. Calculate upper-frequency improvement factor

Calculation: fHf/fH = 500/100

Answer:   ⇒  5

### 24. Calculate overall bandwidth improvement

Original bandwidth: BW = 99.9 kHz

Improved bandwidth: BWf = 499.98 kHz

Improvement factor: BWf/BW = 499.98/99.9

Answer:   ⇒  ≈ 5.005 times

The bandwidth increases by about 400.5% compared with the original bandwidth.

### 25. Find the minimum D required by the cutoff-frequency requirements

For the lower cutoff: 100/D ≤ 25, so D ≥ 4.

For the upper cutoff: 100D ≥ 400 kHz, so D ≥ 4.

Minimum value satisfying both: Dmin = max(4, 4)

Answer:   ⇒  Dmin = 4

### 26. Determine the corresponding minimum β

Using D = 1 + Aβ: 4 = 1 + 100β

Answer:   ⇒  βmin = 0.03

### 27. Can gain and bandwidth requirements be satisfied simultaneously?

Yes, under the stated model. The gain requirement gives β = 0.04 and D = 5. The bandwidth requirements need only D ≥ 4. Therefore, the gain-selected feedback factor also meets both cutoff-frequency requirements: fLf = 20 Hz ≤ 25 Hz and fHf = 500 kHz ≥ 400 kHz.

# Stage 5 – Input and output resistance

The selected voltage-series configuration increases input resistance and reduces output resistance. Using Ri = 10 kΩ, Ro = 10 kΩ and D = 5:

### 28. Calculate Rif

Formula: Rif = RiD = 10 kΩ × 5

Answer:   ⇒  50 kΩ

### 29. Calculate Rof

Formula: Rof = Ro/D = 10 kΩ/5

Answer:   ⇒  2 kΩ

### 30. Calculate percentage change in input resistance

Formula: (Rif − Ri)/Ri × 100%

Substitution: (50 − 10)/10 × 100%

Answer:   ⇒  +400%

### 31. Calculate percentage change in output resistance

Formula: (Rof − Ro)/Ro × 100%

Substitution: (2 − 10)/10 × 100%

Answer:   ⇒  −80%

### 32. Determine whether resistance specifications are satisfied

| Parameter | Required | Calculated | Result |
| --- | --- | --- | --- |
| Input resistance | ≥ 50 kΩ | 50 kΩ | Satisfied at limit |
| Output resistance | ≤ 2 kΩ | 2 kΩ | Satisfied at limit |

### 33. Find the required D for the resistance specifications

For input resistance: 10D ≥ 50, giving D ≥ 5. For output resistance: 10/D ≤ 2, also giving D ≥ 5. Hence the minimum value is D = 5.

### 34. Determine the corresponding β

Calculation: D = 1 + Aβ ⇒ 5 = 1 + 100β

Answer:   ⇒  β = 0.04

### 35. Is the gain-selected feedback factor sufficient?

Yes. The gain specification requires β = 0.04, and the resistance requirements also require D ≥ 5, corresponding to β ≥ 0.04 for A = 100. The gain-derived feedback factor is therefore just sufficient for the resistance targets in the ideal model.

# Stage 6 – Manual design of the final feedback amplifier (15 marks)

### 36. Determine required β

Calculation: β = (A/Af − 1)/A = (100/20 − 1)/100

Answer:   ⇒  β = 0.04

### 37. Determine the feedback network ratio

For a simple voltage divider, define R1 as the resistor from output to the feedback node and R2 as the resistor from the feedback node to ground. Then β = R2/(R1 + R2).

Required ratio: 0.04 = R2/(R1 + R2)

Rearranging: 0.04R1 + 0.04R2 = R2

Result:   ⇒  R1 = 24R2, or R1:R2 = 24:1

This ratio is the ideal target. A practical network is selected next from the permitted resistor values.

### 38. Determine ideal Af

Calculation: Af = 100/[1 + 100(0.04)] = 100/5

Answer:   ⇒  20

### 39. Determine ideal Rif

Calculation: Rif = 10 kΩ × 5

Answer:   ⇒  50 kΩ

### 40. Determine ideal Rof

Calculation: Rof = 10 kΩ/5

Answer:   ⇒  2 kΩ

### 41. Determine ideal lower cutoff frequency

Calculation: fLf = 100 Hz/5

Answer:   ⇒  20 Hz

### 42. Determine ideal upper cutoff frequency

Calculation: fHf = 100 kHz × 5

Answer:   ⇒  500 kHz

### 43. Determine ideal bandwidth

Calculation: BWf = 500,000 − 20 Hz

Answer:   ⇒  499.98 kHz

### 44. Determine ideal desensitivity

Calculation: D = 1 + 100(0.04)

Answer:   ⇒  5

## Practical resistor network

The exact ratio 24:1 can be built using the permitted resistor values by connecting 22 kΩ and 1 kΩ in series for R1, giving R1 = 23 kΩ, and using R2 = 1 kΩ. The total divider resistance is 24 kΩ.

Actual feedback factor: βactual = R2/(R1 + R2) = 1/24

Answer:   ⇒  βactual = 0.041667

Actual desensitivity: Dactual = 1 + 100(0.041667)

Answer:   ⇒  Dactual ≈ 5.1667

Actual closed-loop gain: Af = 100/5.1667

Answer:   ⇒  Af ≈ 19.355

Actual input resistance: Rif = 10 × 5.1667 kΩ

Answer:   ⇒  51.667 kΩ

Actual output resistance: Rof = 10/5.1667 kΩ

Answer:   ⇒  1.935 kΩ

Actual lower cutoff: fLf = 100/5.1667 Hz

Answer:   ⇒  ≈ 19.35 Hz

Actual upper cutoff: fHf = 100 × 5.1667 kHz

Answer:   ⇒  ≈ 516.67 kHz

Actual bandwidth: 516.67 kHz − 0.01935 kHz

Answer:   ⇒  ≈ 516.65 kHz

| Parameter | Ideal target design | Practical divider design |
| --- | --- | --- |
| Β | 0.040000 | 0.041667 |
| Desensitivity D | 5.000 | 5.167 |
| Closed-loop gain | 20.000 | 19.355 |
| Input resistance | 50 kΩ | 51.667 kΩ |
| Output resistance | 2 kΩ | 1.935 kΩ |
| Lower cutoff | 20 Hz | 19.35 Hz |
| Upper cutoff | 500 kHz | 516.67 kHz |
| Bandwidth | 499.98 kHz | ≈ 516.65 kHz |

Practical-design interpretation: The practical network improves the resistance and bandwidth margins, but its gain is approximately 19.355 rather than exactly 20. Thus, if the gain requirement is interpreted as exactly 20, this practical divider does not fully meet every specification. A precision or adjustable feedback network would be needed for closer gain setting.

# Stage 7 – Verification and final engineering decision

### 45. Which specification was most difficult to satisfy?

The most difficult specification is obtaining the exact closed-loop gain of 20 while using only the listed practical resistor values. The ideal feedback factor is 0.04, whereas the chosen available divider produces 0.041667 and a gain of approximately 19.355.

### 46. What is the major advantage of the selected configuration?

Voltage-series feedback provides both a high input resistance and a low output resistance. It also stabilizes gain and extends the usable bandwidth.

### 47. What trade-off is introduced by negative feedback?

The main trade-off is reduced gain. In the ideal design, gain falls from 100 to 20, a reduction of 80%. This reduction is exchanged for improved gain stability, wider bandwidth and better resistance characteristics.

### 48. Is the selected β adequate for all requirements?

The ideal value β = 0.04 is adequate for all requirements in the standard model. The practical value β = 0.041667 meets the input resistance, output resistance and bandwidth limits, but gives a gain of approximately 19.355 rather than exactly 20.

### 49. What happens if β is doubled?

If β is doubled to 0.08, then D = 1 + 100(0.08) = 9.

Gain: Af = 100/9

Answer:   ⇒  11.11

Input resistance: Rif = 10 kΩ × 9

Answer:   ⇒  90 kΩ

Output resistance: Rof = 10 kΩ/9

Answer:   ⇒  1.111 kΩ

Lower cutoff: fLf = 100/9

Answer:   ⇒  11.11 Hz

Upper cutoff: fHf = 100 × 9

Answer:   ⇒  900 kHz

Doubling β increases the desensitivity factor, input resistance and bandwidth and further reduces output resistance. However, the closed-loop gain falls to approximately 11.11, so the gain target is not met.

### 50. How are gain, bandwidth, resistance and desensitivity affected?

| Parameter | Effect when β increases in voltage-series feedback |
| --- | --- |
| Closed-loop gain | Decreases |
| Bandwidth | Increases under the stated single-pole model |
| Input resistance | Increases |
| Output resistance | Decreases |
| Desensitivity factor | Increases |
| Gain stability | Improves |

# Final engineering decision

For the ideal calculation-based design, the decision is YES. With β = 0.04, the gain is 20, input resistance is 50 kΩ, output resistance is 2 kΩ, lower cutoff is 20 Hz and upper cutoff is 500 kHz. Each value meets the required limit.

For the practical resistor network selected from the given list, the strict decision is NO if the gain must equal exactly 20. The practical divider gives Af ≈ 19.355, although its resistance and bandwidth values satisfy the limits. The design should therefore use a precision or adjustable feedback network, or a suitably selected resistor ratio, to bring the actual feedback factor closer to 0.04. The final hardware should be checked because real amplifiers may not follow the ideal model perfectly.

# Overall conclusion

This investigation shows that negative feedback can improve several important amplifier characteristics at the same time, provided that the feedback topology is selected correctly. Voltage-series feedback is appropriate for the stated voltage-amplifier requirements because it increases input resistance and reduces output resistance. With the ideal feedback factor of 0.04, the design achieves the specified gain and extends the bandwidth from 99.9 kHz to approximately 499.98 kHz. The practical resistor example demonstrates that available component values can introduce a small gain error, so practical selection and verification are important parts of engineering design.

# References

1. ECA04 – Analog Circuits, CO1 Investigative Assignment: “Feedback Amplifiers – Analysis and Engineering Investigation,” supplied question paper.

2. Standard negative-feedback amplifier relationships and voltage-divider equations as provided in the course material.